‘WYG’ing with STRAND SHOWNET

The following document details the steps for enabling the WYSIWYG program to interface with a Strand SHOWNET system.  The patch has tested compatible with ‘WYG’ release 7.0.  As of January 2004 this patch is distributed upon request from Strand Lighting (www.strandlight.com ).

The process required is as follows:

1.Copy the strandlx.dll and shownet.reg to the following location:

c:\program files\Strand\

If not already present, create a new directory in program files called Strand.

2.Next double click on the shownet.reg file.  This provides the WYG software the proper file path in the Registry: under the protocols manager section.  Thus the truly technical step is done for you.

3.After that just start WYSIWYG.  Click the LIVE button.

On the TOOL BAR, under LIVE   select DEVICE MANAGER

4.Click NEW.  LIBRARY SELECTION window opens

The following represents the path of selection to designate a Strand Network: NETWORKS>MANUFACTURER>STRAND LIGHTING>STRAND SHOWNET

Click INSERT.

5.The LIBRARY SELECTION window closes, reverting back to DEVICE MANAGER.

Double-click STRAND SHOWNET to verify proper DMX assignment (1, 512, 1024..)

6.Click the CONNECT button. This may have to be done several times to be recognized.  Reboot if repeated attempts fail.

TROUBLESHOOTING:

-Verify your PC is connected to the SHOWNET hub with a discreet IP.

-Rebooting the PC may help.

-Double-check your console or consoles have Networker engaged.

-Double-check your console or consoles have proper NetSlot assignments.

-Double-check all CAT5 connections.